Labconco 2' Purifier Vertical Clean Bench

Catalog Number: 3970200

Overview

Purifier Vertical Clean Benches direct HEPA-filtered air downward over the

work area to provide ISO Class 5 (formerly Class 100) conditions and

protect your work from particulate contamination. Applications for clean

benches include plant tissue culture, media plate preparation, electronics

inspection, medical device assembly and pharmacy drug preparation.

Because they do not provide protection to the user, they should not be

used in conjunction with biohazardous material, toxins or radionuclides.

Purifier Vertical Clean Benches are available in 2', 3' and 4' widths.

Labconco also offers Purifier Horizontal Clean Benches that direct HEPAfiltered

air horizontally over the work area and Purifier Filtered PCR

Enclosures, which are specialized vertical clean benches designed for

polymerase chain reactions.

Specifications

Specifications

Weight: 125.0 lbs

Weight metric: 57.0 kg

Dimensions: 24" w x 28.6" d x 36.8" h

Electrical: 115 volts, 60 Hz, 5 A

Laminar Flow: Vertical

Nominal Width: 2'

Conformance: CAN/CSA C22.2, UL

Power Cord & Plug: North America, 115V, 15A

Features

Nominal downflow velocity of 45-65 fpm

Aerodynamic air foil

Supply HEPA filter, 99.99% efficient on particles 0.3 micron

ISO Class 5 air (formerly Class 100 per Federal Standard 209E)

Disposable prefilter

Variable speed built-in blower with motorized impeller and solid state control rated for 10 amps

Intrinsically-sound, negative pressure design

Minihelic pressure gauge

Air diffuser

UV-resistant, glacier white and gray, powder-coated aluminum and steel construction

UV-opaque and resistant 1/4" thick tempered safety glass sash and sides

Fluorescent lamp

Light and blower switches

9.4" high sash opening

Ergonomic 10° angled and hinged sash that pivots upward, locks to a loading height of 20" and has

wiping seals to promote ISO Class 5 conditions

Two utility ports with iris openings and plugs

ETL Listing (115 volt, 60 Hz models)

One year warranty on parts and service

Features

Nominal downflow velocity of 45-65 fpm

Aerodynamic air foil

Supply HEPA filter, 99.99% efficient on particles 0.3 micron

ISO Class 5 air (formerly Class 100 per Federal Standard 209E)

Disposable prefilter

Variable speed built-in blower with motorized impeller and solid state control rated for 10 amps

Intrinsically-sound, negative pressure design

Minihelic pressure gauge

Air diffuser

UV-resistant, glacier white and gray, powder-coated aluminum and steel construction

UV-opaque and resistant 1/4" thick tempered safety glass sash and sides

Fluorescent lamp

Light and blower switches

9.4" high sash opening

Ergonomic 10° angled and hinged sash that pivots upward, locks to a loading height of 20" and has

wiping seals to promote ISO Class 5 conditions

Two utility ports with iris openings and plugs

ETL Listing (115 volt, 60 Hz models)

One year warranty on parts and service

Options

Built-in Guardian Airflow Monitor that continuously monitor downflow airflow and has two LED lights to indicate when airflow is within or outside set point range.

254 nm UV lamp with 3-way switch (fluorescent, UV and off) and protection panel interlocked with the

UV lamp that closes the sash opening

2', 3' and 4' widths

International electrical configurations

Required Accessories

Supporting work surface. See Solid Epoxy Dished Work Surfaces

Supporting base. See Adjustable Height Base Stands

Optional Accessories

General Accessories

IV Bar Kit

Compliance

UL Standard 3101-1/61010-1 (115 volt models)

CAN/CSA Standard C22.2 No. 1010.1 conformity (115 volt models)

CE Conformity marking (230 volt models)